General instructions on how to use the AD9081 with the ZCU102

Jon.Kraft@analog.com, Dec 2023

Note:  These were created, and verified to work, in Dec 2023.  Things may have changed a bit since that time...  But the general flow should be the same

- Install the LIBIIO drivers

  - These might already be on your computer (if you can run IIOscope, then these are probably on your computer)

  - Otherwise, follow instructions here:  https://wiki.analog.com/sdrseminars

- Download the FPGA's SD Card here:

  - https://wiki.analog.com/resources/tools-software/linux-software/kuiper-linux

    - At the time of this guide, 2021_r2 release image was used.

  - You can burn image files using Win32DiskImager, on an ADI computer.  It won't corrupt the SD card.

- Move the AD9081/ZCU102 files on the SD card:

  - You must do this on a non-ADI computer!  Moving these files  on an ADI computer will corrupt the SD card!

  - https://wiki.analog.com/resources/eval/user-guides/ad9081_fmca_ebz/quickstart/zynqmp#sdcard_boot_files

  - Copy these files to the SD card root:  Image, BOOT.BIN, and system.dtb

    - Copy the Image file from the zynqmp-common directory.

      - Note:  it is the "zynqmp-common" directory, not the "zynq-common" directory

    - Copy the BOOT.BIN and system.dtb from the zynqmp-zcu102-rev10-ad9081-m8-l4 directory.

      - For evaluation boards populated with VXCO 100 MHz copy the device tree from vcxo100 folder.

      - For evaluation boards populated with VXCO 122.88 MHz copy the device tree from vcxo122p88 folder.

      - The VCO is the large silver "box" on the back of the AD9081/2 eval board.  Just read the label on it.

- Configure ZCU102 board like this:

  - https://wiki.analog.com/resources/eval/user-guides/ad9081_fmca_ebz/quickstart/zynqmp#testing

  - Connect SMA cable, with a 20dB attenuator, between Rx0 and Tx0

  - See photo below of the completed setup

- Using a USB to Ethernet adapter, configure a static IP

  - Go to network properties on your Windows computer, find that Ethernet device, right click and select "Properties"

  - We'll set a static IP connection, so make sure that the IP address field is 192.168.0.2

- Configure the IP address of the FPGA board

  - Power up the ZCU102 FPGA board, and wait about a minute

  - Open up device manager on your laptop, select Ports, and find the "Silicon Labs Quad xxxxx:  Interface 0

- Now open the application "PuTTY" and enter in that COM port, Speed=115200, Connection Type=Serial

- At the "root@analog:" command prompt type:  enable_static_ip.sh 192.168.0.21

- Now type:  ifconfig eth0 192.168.0.10

- Let's find out what's connected to the ZCU102!

  - Type:  iio_attr -a -d

- This should show you have an "AD9081-FMCA-xxxx" connected.  And that it has iio devices for HMC7044, 9081-tx, and 9081-rx.  See above image.

- If you don’t see this, there is something wrong with your eval board (or it's not plugged in!)

- Now open IIO Oscilloscope (osc.exe) on your windows computer

  - Go to Settings --> Connect

  - Select "Manual" and enter:  ip:192.168.0.10

  - Click "Refresh"

- Then click "Connect"

- Now let's configure the AD9081 to output 2 tones (3 MHz and 5 MHz) at 750MHz:

- Hit "Refresh" to make sure all the values are accepted.

- Then Select File=-->New Plot, click on "voltage0_i", "voltage0_q".  Plot type = "Frequency Domain", then play button:

- Ok, if you can do all of that, then we're ready to start programming!

  - If you can't do that, then DO NOT proceed further!  You've got to get this much working.

  - But be encouraged!  Establishing the connection is often the most frustrating part of the whole process.  And you only need to do this setup once--everything will be remembered next time you power up.  So once you get it working, it'll only get more fun!

- Now you are ready to connect to Matlab or Python.  See the other docs for help with that!